## Abstract

Mobile networks carry each subscriber's internet traffic through tunnels called GTP-U (GPRS Tunnelling Protocol, user plane). On the link between the radio network and the core, called N3 in 5G and S1-U or Iu in 4G and 3G, this traffic usually travels as plain UDP on port 2152, with no authentication or encryption between network elements. Anyone who gains a foothold on that link can therefore abuse it. That foothold could be a compromised or rogue base station, a misconfigured roaming connection, or an insider with network access.

This paper presents gtpu-abuse-lab, a small and fully open testbed built with Docker Compose. It combines a real Open5GS 5G core, a real UERANSIM-simulated base station and handset, a passive detector written with Scapy, and a generator that produces labelled abuse traffic. The detector applies four rules: nested GTP tunnels (GTP-in-GTP), TEID spoofing, control-plane messages hidden inside user-plane traffic (PFCP and NGAP), and inner packets aimed at core network functions.

We evaluate the detector on a 1,320-packet labelled corpus. Its 720 benign packets span twelve traffic types, including deliberately awkward cases such as non-IP "Unstructured" payloads and legitimate handovers. On this corpus the detector reaches precision, recall, and F1 of 1.0 with a false-positive rate of 0.0, and the result stays the same across five random seeds. The whole evaluation runs offline with no special infrastructure. It also does three things that a single score cannot: it compares against a naive detector that misses every nested tunnel, it traces each remaining false positive to one cause (legitimate handovers) and then removes it, and it lists the attacks the detector cannot catch in an explicit evasion suite.

We then validate the same detector against a genuinely live 5G core and radio inside Docker. A 500-packet labelled corpus was sent over a raw socket rather than replayed from a file, and all four rules fired on it. The live findings stream matched an offline pass over the same packets finding for finding, so the run could be scored packet by packet. Scored that way, it gave 608 findings with precision 0.960, recall 0.9625, and a false-positive rate of 0.16, and every error traced back to a defect in the TEID-spoofing rule. With the rule fixed, a second live run of the same packet sequence gave 550 findings, precision and recall of 1.0, and a false-positive rate of 0.0. A live figure published in an earlier version of this paper (131 findings) did not reproduce and is withdrawn.

Finally, the paper reports a lesson about testing. Bringing the live system up for the first time exposed five real bugs, including one where the traffic generator had been sending malformed Ethernet frames on every live run since the project began. The labelled live run exposed two more: the handset's traffic had never reached the data network although the documented check for it passed, and the TEID-spoofing rule let a spoofer take a tunnel over. None of these seven bugs showed up in the existing, fully passing offline tests. Six sit in code those tests never run, and we argue this is structural rather than accidental: for tools that work on raw network bytes, an in-memory or file-replay test suite is necessary but not sufficient, because the bugs that matter most only appear when bytes are actually put on, or read off, a real wire. The seventh needed only a realistic packet order, which the live corpus supplied and a file replay now reproduces.

## 1. Introduction

### 1.1 Motivation

Mobile core networks move user data through GTP-U tunnels. Each tunnel has a Tunnel Endpoint Identifier (TEID), a number that identifies it locally. The protocol does not cryptographically tie a TEID to its claimed source or to the subscriber session it represents. Production networks make up for this with physical and network isolation: GTP-U is expected to cross only trusted infrastructure, such as links between the radio network and the core, or roaming interconnects secured by other means. That isolation is a deployment assumption, not a guarantee built into the protocol, and it has failed in practice at the interconnect layer for related protocols in the same family, notably SS7 and Diameter.

A party with access to the transport segment can attempt several GTP-specific abuses:

- Nested tunnels (GTP-in-GTP): placing a second GTP header inside what should be an ordinary IP payload. Depending on how downstream equipment or a naive detector reads the packet, this can smuggle traffic past inspection points that only look one layer deep, or pivot into infrastructure that trusts the inner tunnel.

- TEID spoofing: sending traffic under a TEID that belongs to another subscriber's session, from an unexpected source address. This can hijack, inject into, or deny service to that session.

- Control-plane smuggling: hiding control-plane traffic (PFCP on N4, GTP-C, or NGAP/S1AP normally carried over SCTP on N2) inside GTP-U user-plane payload, to reach or influence control-plane functions through a data path that may be watched less closely.

- Inner-traffic misdirection: putting an inner IP packet inside a normal-looking tunnel but addressing it to an internal core function (UPF, SMF, or AMF) instead of the external internet, using the tunnel to reach systems the subscriber should never be able to route to.

Commercial "GTP firewall" products are sold against exactly these abuses, but they are closed appliances. Their detection logic, false-positive behaviour, and performance cannot be checked independently, and no small, open, reproducible testbed lets a researcher try a detection approach against a real (simulated) 5G core instead of a static packet capture of unknown origin.

### 1.2 Contributions

- An open, Dockerized, reproducible testbed that combines a real Open5GS 5G core and a UERANSIM radio and handset simulator on an isolated 10.10.10.0/24 network under the test PLMN 999/70, together with a passive detector and a labelled attack-traffic generator (Section 4).

- A four-rule passive detector (Section 5), evaluated rigorously rather than circularly (Section 6.1). On a 1,320-packet corpus whose 720 benign packets span twelve traffic types, including awkward Unstructured-PDU bytes and legitimate handovers, it reaches precision, recall, and F1 of 1.0 with a false-positive rate of 0.0, stable across five seeds. The evaluation adds three things a bare score omits: a naive-baseline comparison that measures the contribution, a false-positive analysis that traces every remaining false alarm to one cause and then removes it, and an evasion suite that states the detector's blind spots plainly. All of it runs with a single pip install scapy pytest and no root or Docker.

- A methodological finding built into the detector (rule R1, Section 5.1). Scapy's default parsing of a GTP-U payload never decodes a nested GTP header as GTP: it hands the bytes back as Raw or, inside a realistic outer G-PDU, mis-reads them as PPP. A naive haslayer() check on captured or live traffic therefore misses 100% of nested-tunnel abuse, even though an in-memory unit test that builds the layers directly in Python would pass. Section 6.1.3 measures this: an otherwise identical naive detector scores 0.0 recall on nested tunnels where the real detector scores 1.0.

- Live validation of the same detector against real, transmitted (not replayed) 5G traffic (Section 6.2), confirming that the same rules fire as in the offline test, and a per-packet scoring of that live run against its labels. The live findings match an offline pass over the same packets finding for finding.

- A documented case study (Section 8, with full detail in FINDINGS.md) of seven real bugs. Five were found at first bring-up: a missing runtime dependency, a config-patching regex that quietly does nothing while logging success, a lost file permission in a multi-stage container build, a Docker Compose networking trap, and, most importantly, a traffic generator that had been sending malformed Ethernet frames on every live run since the project began. Two more were found during revision by scoring a live run packet by packet: the UE data path had never worked while the documented check for it passed, and rule R2 handed a TEID to whichever source sent last. All seven were found only by running the live path. Six of them sit in code the offline tests never run; the seventh sits in code they do run, but needs a packet order their corpus never produces.

## 2. Background

### 2.1 GTP-U and its role in the mobile core

GTP-U (defined in 3GPP TS 29.281) carries user traffic inside UDP, normally on port 2152. In 5G the interface this paper focuses on is N3, the link between the gNB (the base station) and the UPF (the User Plane Function). N3 carries every subscriber's actual data, and each session's tunnel is identified by a TEID that the receiving end chooses when the tunnel is set up. Two neighbouring interfaces matter for the threat model:

- N2 carries signalling between the gNB and the AMF, using NGAP over SCTP. It handles registration, authentication, and session setup, and is supposed to stay strictly separate from N3's user traffic.

- N4 carries signalling between the SMF and the UPF, using PFCP (3GPP TS 29.244). It controls how the UPF forwards a tunnel's traffic, and is also supposed to stay separate from N3.

The abuses listed in Section 1.1 are all attempts to blur or bypass this separation, or to exploit the fact that GTP-U does not cryptographically bind a TEID to a subscriber.

### 2.2 Threat model

The testbed assumes an attacker who can inject arbitrary UDP packets onto the N3 segment. This is the position a compromised or rogue gNB, a misconfigured roaming partner, or a network insider would hold. In the live lab this position is played by a separate attacker container that writes onto the core's N3 interface directly; it is not a UE and holds no PDU session (Section 4.3). The threat model does not assume an attacker who has taken over the UPF or SMF itself, which is a stronger and different problem, and it does not cover radio-layer attacks on the air interface. The detector is a passive tap: it watches N3 traffic and raises findings, but it does not block or change traffic. This matches where a production GTP firewall usually sits, in front of or spanning the UPF's N3 interface rather than in the UPF's own forwarding path.

## 3. Related work

Abuse of mobile interconnects has a long record at the SS7 and Diameter layers, including location tracking, fraud, and interception attacks disclosed over the past decade. Industry bodies such as the GSMA publish security guidance for GTP as part of their wider mobile-security work. Commercial GTP and signalling firewalls from telecom security vendors target this abuse directly, but they are closed appliances with no independently reproducible detection logic or benchmark corpus.

The set of abuse classes used by this project's traffic generator (attacker/generate_attacks.py) was designed with reference to prior published discussion of GTP tunnel-endpoint abuse, as noted in the tool's own comments. We have not independently traced that taxonomy to a single citable, permanent source, and we do not invent a specific citation here; we would rather flag the gap than fabricate a reference. Whatever the taxonomy's exact provenance, what this project adds is a runnable, open implementation of the detectors, the labelled attack traffic, and a real (simulated) 5G core to test both against. That combination is rarely available together in either the commercial or the academic literature.

## 4. System architecture

### 4.1 Components

| Component | Role |
| --- | --- |
| core/ | Open5GS all-in-one 5G core (AMF, SMF, UPF, plus support functions: NRF, SCP, AUSF, UDM, UDR, PCF, BSF, NSSF) |
| ran/ | UERANSIM-simulated gNB and UE, generating real GTP-U traffic on N3 |
| detector/ | The main research contribution: a passive Scapy-based detector, the rule engine, metrics, and the naive baseline (baselines.py) used to measure the contribution |
| attacker/ | Lab-only traffic: the abuse generator, the realistic twelve-category benign generator (benign_traffic.py), and the crafted evasion suite (evasions.py) |
| eval/ | Reproducible offline benchmark (benchmark.py) producing metrics.json and RESULTS.md: headline metrics, baseline comparison, false-positive analysis, multi-seed stability, and evasion scoring |

### 4.2 Topology

UE (uesimtun0)                  gNB (UERANSIM)
 10.45.0.x ── PDU session ──►    10.10.10.20
                                      │
   ┌──────────────────────────────────┘  N3 / GTP-U, UDP 2152
   ▼
 ┌─ core network namespace (10.10.10.10) ─────────────────────────────┐
 │ eth0 ──► UPF (Open5GS) ──► ogstun 10.45.0.1/16 ──► NAT ────────────┼──► data network
 │  │ ▲                                                               │
 │  │ └── ATTACKER (lab only): crafted GTP-U to the UPF               │
 │  │     over a raw socket; not a UE, no PDU session                 │
 │  ▼                                                                 │
 │ DETECTOR (tap): sniffs eth0 passively, udp port 2152               │
 └────────────────────────────────────────────────────────────────────┘

The legitimate data path runs from the UE through the gNB as uplink GTP-U to the UPF, which decapsulates it onto its tunnel device ogstun, from where the core's NAT rule sends it out. Replies return as downlink GTP-U. The attacker is not on this path: it injects its own GTP-U on the core side, where the detector also listens.

All components run on a fixed private Docker network (10.10.10.0/24) under the test PLMN 999/70. This is never a real operator's identity and is never meant to be reachable outside the isolated lab (see Section 11, Ethics).

### 4.3 Why the detector shares the core's network namespace

Traffic between two containers on a Docker bridge is switched at Layer 2, so a third container on the same bridge does not see it by default. The docker-compose.yml file sets detector: network_mode: "service:core", which places the detector inside the UPF's own network namespace and lets it sniff the real eth0. This is deliberately the same tap point a production GTP firewall uses, in front of or spanning the UPF's N3 interface, rather than a port-mirroring approximation. This choice has an operational consequence, documented in Section 8 and in FINDINGS.md as Finding 5.

The attacker service uses the same network_mode: "service:core", so its frames go out on the interface the detector sniffs. It is not a UE and holds no PDU session. It writes crafted GTP-U with a raw socket onto the core's eth0, the N3 interface, addressed to the UPF, and UERANSIM does not encapsulate these packets. It stands in for the on-path injector of the threat model (Section 2.2). Whether the UPF acts on the crafted frames was not measured; the detector observes them on eth0 as a tap would.

### 4.4 Two operating modes

- Offline evaluation, with no Docker, core, or root. The attack generator writes a labelled packet capture, the detector replays it, and a scoring harness produces metrics.json and a markdown results table. This is the reproducible path behind the headline numbers in Section 6.1, and it runs with pip install scapy pytest alone.

- Live lab, the full Docker stack, which produces genuinely transmitted (not replayed) 5G traffic and is used for the validation in Section 6.2.

Both modes use the exact same detector/rules.py and detector/metrics.py code. There is one detection engine, not two, so results from the two modes are directly comparable in kind (Section 6.3).

## 5. Detection methodology

The detector (detector/rules.py) applies four rules. Each rule is a small, self-contained function of the form rule(pkt, state) -> list[Finding], and a shared evaluate() dispatcher runs all four over every GTP-U packet. The dispatcher catches and discards any exception from an individual rule, so a single malformed packet can never crash the detector (verified by test_detector_survives_garbage). The DetectorState object holds the small amount of memory the rules need between packets: a map from each (receiving address, TEID) pair to the source that owns it (for R2), an optional set of known gNB addresses (for R2's handover exception), and a configured set of core-function IP addresses (for R4).

### 5.1 Rule R1: nested tunnels (GTP-in-GTP), severity critical

R1 catches a second GTP header hidden inside a payload that should contain plain user IP data. In other words, a tunnel inside a tunnel.

The interesting part, and this project's main technical contribution, is how easily a passive detector misses this. When Scapy parses the payload of a GTP-U packet, it looks at the first four bits to decide what the inner data is: a value of 4 means an IPv4 packet, and 6 means IPv6. A nested GTP header starts with neither. Its first byte falls in the range 0x30 to 0x3f (GTP version 1, with the protocol-type bit set), so Scapy never decodes the inner payload as GTP. It hands the bytes back as a non-IP layer instead, Raw or PPP depending on the outer header (see below). A detector that checks pkt[GTP_U_Header].payload.haslayer(GTP_U_Header) on captured or live traffic therefore gets False for every real nested-tunnel packet. The abuse is present; Scapy's own parsing heuristic has simply thrown away the information needed to see it.

This gap is invisible to a naive unit test. If a test builds the nested packet directly in Python, for example GTP_U_Header(...)/GTP_U_Header(...)/IP(...), the inner GTP_U_Header already exists as a real Scapy layer in memory, haslayer() finds it at once, and the test passes. The test never puts the packet through the serialize-to-bytes and parse-from-bytes round trip that real captured traffic goes through, so it cannot notice that parsing, as opposed to the in-memory object, loses this information.

The fix (_looks_like_gtp(), _reparse_inner(), and _carries_inner_ip() in rules.py) actively re-reads the inner payload's raw bytes whenever the default parse did not already yield a routable inner (an IPv4 or IPv6 packet) or a recognised GTP header. This detail matters more than it first appears. Scapy does not simply hand a nested GTP header back as Raw: which class it guesses depends on the outer message type. Under a bare outer header with message type 0x00, the packet decodes as IP / UDP / GTP-U / Raw. Inside a realistic outer G-PDU (message type 0xFF), the form a UPF actually decapsulates and the form the corpus uses, it decodes as IP / UDP / GTP-U / PPP / Raw: the first bytes of the nested header are read as a PPP header. In neither case does a GTP layer appear. Keying the re-parse on the Raw class alone would catch only the packets that happen to be left as Raw; keying it on the bytes, as we do, catches the nested tunnel regardless of which wrong class the default parser assigned. The engine keeps a payload that Scapy already decoded as IPv4, IPv6 or a GTP header. Anything else is serialised back to bytes as a whole layer (bytes(inner)), not just its trailing load, because a PPP guess has already consumed the start of the nested header. If the first byte's top three bits indicate GTP version 1 with the protocol-type bit set, and the second byte is a known GTP message type (a G-PDU, an echo request or response, and so on), the bytes are parsed again as a GTP header before the rule checks for nesting.

Two extra guards stop this from raising false alarms on harmless non-IP payloads. (5G "Unstructured" PDU sessions carry arbitrary bytes, and some of them happen to match the GTP byte pattern.) First, the 16-bit GTP length field must match the bytes actually present, so that 8 + length equals the datagram size apart from padding. Second, and decisively, the nested header must actually carry a routable inner IP packet, recognisable by an IPv4 or IPv6 version nibble. A tunnel used for abuse forwards a real packet; random noise does not. Section 6.1.4 shows that these guards remove the Unstructured-PDU false positives completely (zero across 200 crafted payloads), and Section 6.1.3 shows they cost nothing in detection.

As far as we know, this is an underdocumented requirement for any Scapy-based passive GTP-U tool, and it generalises. Any passive detector tested only against packet objects built in memory, rather than against bytes parsed back off the wire, risks silently missing exactly the abuse that depends on a parser's fallback behaviour. Section 8 develops this point further, using seven more concrete cases found on this project's own live path.

### 5.2 Rule R2: TEID spoofing, severity high

R2 catches a TEID that was first seen from one source IP and later arrives at the same receiving endpoint from a different source IP.

The implementation is deliberately simple. A TEID is unique only within the endpoint that receives it (3GPP TS 29.281), so the same number on the UPF's uplink and on a gNB's downlink names two different tunnels. DetectorState.teid_owner is therefore keyed on the pair (receiving address, TEID), where the receiving address is the destination of the outer IPv4 or IPv6 header that carries the tunnel, and records the first source seen for each pair; that source becomes the owner. Any later packet on the same pair from a different source raises a finding and leaves ownership where it was, so a rogue cannot take a TEID over by sending on it. This first-source-wins approach trades the ability to catch sophisticated, session-aware spoofing for something auditable and easy to unit-test. An earlier version keyed ownership on the TEID alone and moved it to whichever source sent last. The labelled live run exposed that defect (Section 8, item 7).

There is one honest complication: handovers. A legitimate Xn or N2 handover moves a handset from one base station to another. The UPF's uplink TEID stays the same, but user-plane packets now arrive from a different gNB IP. Looking at the user plane alone, that is indistinguishable from a spoof, so a naive version of R2 raises a false alarm on every handover. Section 6.1.4 quantifies this: without any mitigation, every remaining false positive is a handover.

The mitigation is an optional allowlist of known gNB IPs, supplied by the operator (--gnb-ips, stored in DetectorState.known_gnb_ips). If a packet arrives from a new source that is a known gNB, R2 lets it take ownership and raises no finding. That covers a handover, and it also lets the real gNB reclaim a tunnel a rogue was seen on first, for example after a detector restart, instead of being flagged on every packet. A packet from any address outside the pool still fires, and ownership does not move. Without the allowlist every change of source is flagged and ownership never moves, so a handed-over session keeps raising R2 for as long as it sends from the new gNB, not just once at the handover. With the allowlist, handover false positives drop to zero while every rogue-source spoof in the corpus (teid_spoof, recall 1.0, Section 6.1.3) is still caught. This is a deliberate, honest treatment of a real limitation of stateless passive attribution, rather than a corpus that quietly leaves handovers out. Two blind spots remain: a rogue that forges a legitimate gNB's source IP, and a rogue that claims a TEID the detector has never seen. Both are listed as evasion cases in Section 6.1.5 and in Section 9. Closing either one needs control-plane correlation, which is outside the scope of a user-plane-only tap.

### 5.3 Rule R3: control-plane smuggling, severity critical

R3 catches control-plane traffic hidden inside a GTP-U user-plane payload: PFCP (N4, UDP port 8805), GTP-C (UDP port 2123), GTP-U (UDP port 2152, kept here as defence in depth alongside R1), or SCTP (protocol 132, which carries NGAP and S1AP). These are control protocols appearing where only user data belongs. The rule inspects the inner packet's transport protocol and port, re-parsing the inner bytes first where needed (as in Section 5.1), and compares them against this known-bad set.

### 5.4 Rule R4: inner traffic aimed at a core function, severity high

R4 catches an inner IP packet, inside an otherwise normal-looking tunnel, whose destination is a configured core-function address (UPF, SMF, AMF, and so on) rather than the external data network. The implementation is an operator-configured allowlist (DetectorState.core_nf_ips, set with --core-ips when the detector starts). This is a deliberate trade-off: the rule is only as complete as its configuration, and a missing or wrong entry quietly disables the rule for that address rather than raising an error (see Section 9).

## 6. Evaluation

The full raw data, JSON artifacts, and verbatim log excerpts for this section are in RESULTS.md, and the exact commands to reproduce every number are in REPRODUCE.md.

### 6.1 Offline evaluation (comprehensive benchmark)

The evaluation is built to answer the questions that a perfect score on a self-generated corpus would otherwise invite. It runs from one reproducible command (python3 eval/run_eval.py, with no Docker, root, or network) and produces seven distinct pieces of evidence, all written to RESULTS.md.

The corpus has 1,320 packets on the primary seed. 600 are malicious, spread evenly across the five attack classes. The other 720 are benign and span twelve traffic types: TLS, HTTP, DNS, QUIC, NTP, RTP/VoIP, ICMP, IPv6, fragmented IP, and IP-options, plus two types chosen specifically to stress the detector. The first is Unstructured-PDU-session bytes, non-IP inner payloads, some crafted to begin with GTP-looking bytes. The second is legitimate Xn/N2 handovers, where the uplink TEID is unchanged but the source gNB IP is different. The earlier evaluation used a single benign shape (ICMP to 8.8.8.8); it is replaced entirely, because a false-positive rate only means something when it is measured against varied benign traffic.

#### 6.1.1 Headline classification (robust detector)

| Metric | Value |
| --- | --- |
| Precision | 1.0 |
| Recall | 1.0 |
| F1 | 1.0 |
| False-positive rate | 0.0 |
| Mean per-packet latency | about 686 µs (dissection and rules together) |
| Implied single-core throughput | about 1,450 packets per second |

#### 6.1.2 Stability across seeds

The headline metrics are not the product of one lucky corpus. Across seeds {1337, 1, 2, 3, 4}, precision, recall, F1, and the false-positive rate each have a mean of 1.0 and a standard deviation of 0.0.

#### 6.1.3 Naive-baseline comparison (the contribution, measured)

To measure what the R1 re-parse is worth, we implement the detector a competent engineer would write first: Scapy's default parsing with haslayer and no inner re-parse (detector/baselines.py). We score it on the identical corpus, holding every other design decision fixed, so the difference isolates one variable, the re-parse.

| Attack class | Robust recall | Naive recall |
| --- | --- | --- |
| gtp_in_gtp | 1.0 | 0.0 |
| pfcp_smuggle | 1.0 | 1.0 |
| ngap_smuggle | 1.0 | 1.0 |
| inner_to_core | 1.0 | 1.0 |
| teid_spoof | 1.0 | 1.0 |
| overall | 1.0 | 0.80 (F1 0.889) |

The naive baseline misses 100% of nested tunnels, because by default a nested GTP header is never parsed as GTP; in the corpus's realistic G-PDU form Scapy reads it as PPP (Section 5.1). This makes the claim in Sections 1.2 and 5.1 concrete: the contribution is worth moving R1 recall from 0.0 to 1.0, and overall F1 from 0.889 to 1.0.

#### 6.1.4 Where the false positives come from, and the handover ablation

Against the corpus above, the robust detector's only remaining source of false positives is the handover category, because a legitimate source-IP change on an unchanged TEID looks exactly like a spoof on the user plane. We report this as an explicit before-and-after rather than hiding it:

| R2 configuration | FPR | False positives | Source |
| --- | --- | --- | --- |
| with known-gNB allowlist (--gnb-ips) | 0.0 | 0 | none |
| without allowlist | 0.0083 | 6 | 100% handover |

Supplying the operator's known gNB addresses removes the handover false positives while still flagging a TEID re-sourced from any address outside the pool. The other earlier source of false positives, benign Unstructured-PDU bytes that happened to match the GTP pattern, was removed at the root by checking the nested header's length field and requiring a routable inner IP (Section 5.1). It now produces zero false positives across 200 crafted Unstructured payloads (test_benign.py).

#### 6.1.5 Adversarial robustness (evasion suite)

The detector is scored against 11 crafted evasions (attacker/evasions.py). Each one is labelled in advance with whether a correct detector should catch it, or whether it is an inherent blind spot of a stateless passive rule. Six are robustness wins, all still caught: an IPv6 inner tunnel, an outer extension-header offset shift, generic-GTP nesting, standard-port PFCP, SCTP/NGAP, and targeting a listed core function. Four are documented blind spots: a rogue that owns a never-seen TEID, a forged gNB source IP, PFCP moved off its usual port, and a core function outside the configured set. The remaining case is correct silence rather than a blind spot: GTP-looking noise that carries no routable inner IP, which the detector rightly leaves unflagged. The suite checks that observed behaviour matches the labelled expectation on every case, so a change in either direction fails the build.

All of Section 6.1 is reproducible with a single pip install scapy pytest and no Docker, root, or network, through python3 eval/run_eval.py.

### 6.2 Live full-stack validation

A real Open5GS core and a UERANSIM UE and gNB were brought up in Docker. The UE completed NG Setup, NAS registration, and establishment of an IPv4 PDU session, producing a real uesimtun0 interface at 10.45.0.2. The command docker compose exec ran ping -I uesimtun0 -c3 8.8.8.8 then confirmed the UE data path end to end. A tcpdump on the core showed each echo request arrive as uplink GTP-U from the gNB (10.10.10.20) to the UPF (10.10.10.10), the inner ICMP appear on the UPF's tunnel device ogstun and leave through NAT, and each reply return as downlink GTP-U from 10.10.10.10 to 10.10.10.20. All 3 replies were received. The session's TEIDs were 0xb2aa uplink and 0x1 downlink. The detector counted these packets and raised zero findings, which is the right outcome for this traffic. An earlier version of this paper cited a ping run through the nr-binder helper as the same proof; that ping never entered the tunnel (Section 8, item 6).

A 500-packet labelled attack corpus was then sent live over a raw socket by the attacker container, which is not the UE (Section 4.3). The corpus is seeded (1337) and is the same on every run: 100 benign, 88 teid_spoof, 82 gtp_in_gtp, 80 inner_to_core, 79 pfcp_smuggle and 71 ngap_smuggle packets. Every packet uses TEID 0x1 and outer destination 10.10.10.10. The outer source is 10.10.10.20, except for the teid_spoof packets, which come from 10.10.10.66. The send takes about 17 minutes, because Scapy cannot resolve a MAC address for the UPF from inside the core's own namespace and sends each frame as a broadcast. The passive detector, running in the same namespace, saw all 500 packets and did not crash.

To score the run packet by packet, the same corpus is written to a capture file with its labels and passed through the detector's offline pcap mode (make live-score). For the live run reported below as "before the R2 fix", the live findings stream was identical, in order and content (rule, source, destination, TEID and detail), to that offline pass, for all 608 of its findings. The per-packet scores therefore apply to the live run. The scoring exposed a defect in R2 (Section 8, item 7). Before and after the fix:

|  | Before the R2 fix | After the R2 fix |
| --- | --- | --- |
| R1_GTP_IN_GTP | 82 | 82 |
| R2_TEID_SPOOF | 146 | 88 |
| R3_CP_SMUGGLING | 150 | 150 |
| R4_INNER_TO_CORE | 230 | 230 |
| Total findings | 608 | 550 |
| True / false positives | 385 / 16 | 400 / 0 |
| False / true negatives | 15 / 84 | 0 / 100 |
| Precision | 0.960 | 1.0 |
| Recall | 0.9625 | 1.0 |
| F1 | 0.961 | 1.0 |
| False-positive rate | 0.16 | 0.0 |

The before column is a live run on a freshly started lab; the after column is a live run on a freshly recreated detector. In the after run the fixed detector saw all 500 packets and raised 550 findings, and its findings stream was identical in order and content to the offline pass. Four UE pings through the tunnel before the attack and four after it (40 GTP-U packets in all) raised no finding, and the detector counted 540 packets, 500 + 40. The unit test test_live_attack_corpus_scores_clean locks the after-fix per-packet verdicts (every malicious packet flagged, every benign packet clean). Findings exceed the 400 malicious packets because rules overlap: every pfcp_smuggle and ngap_smuggle packet raises both R3 and R4, since its inner destination is the UPF. Every error before the fix came from R2. All 16 false positives were a benign packet immediately after a teid_spoof: ownership had moved to the rogue, so the gNB's own packet looked like the spoof. All 15 false negatives were a teid_spoof immediately after another teid_spoof: the rogue already owned the TEID, so nothing had changed. After the fix R2 fires exactly once for each of the 88 teid_spoof packets and never on the gNB, and R1, R3 and R4 are unchanged.

The live figure published in an earlier version of this paper, 131 findings (R1 22, R2 17, R3 37, R4 55), did not reproduce and is withdrawn.

### 6.3 Coverage parity

The same four rules fired in both the offline corpus (Section 6.1) and the live run (Section 6.2). The counts differ (120, 120, 240 and 360 offline for R1 to R4; 82, 88, 150 and 230 on the live corpus after the fix) because the two corpora differ in size and composition, but the coverage is the same. On the identical corpus the agreement is exact: the live findings stream and an offline pass over the same 500 packets match finding for finding. Since both paths share one detection engine (Section 4.4), this is evidence that the offline numbers reflect genuine detection ability rather than a corpus quietly shaped to fit the detector's assumptions.

### 6.4 Consistency across dissectors

If the nested-tunnel blind spot were a quirk of one library, the contribution would be narrow. Three checks argue it is not.

First, across Scapy versions: the offline path ran under Scapy 2.4.4 on the host and the live path under Scapy 2.7.0 in the container, and R1 fired under both.

Second, against an independent dissector. We wrote a three-packet probe capture (attacker/dissector_probe.py): a bare-nested abuse packet, a benign packet with an inner IP payload, and a control-plane-smuggle packet, all carried inside a valid outer G-PDU. Read with Wireshark's tshark, all three frames are recognised as GTP-U on the outer tunnel, but the nested frame's protocol stack ends at that outer GTP-U: its inner bytes are absorbed as an opaque T-PDU, no second GTP layer and no inner IP appear, so a detector that asks whether a G-PDU's payload is itself GTP-U learns nothing. The benign and smuggle frames, by contrast, decode to their inner IP exactly as in Scapy.

Third, against Zeek, a third and independently written network monitor. On the same probe, Zeek records exactly one GTPv1 tunnel, the outer one, and never a nested second tunnel. It decapsulates and logs the inner connections of the benign packet (an ICMP flow to the data network) and the smuggle packet (a UDP flow to the PFCP port) through its tunnel-parent field, but the nested packet produces no inner connection and no second tunnel: Zeek's GTPv1 analyzer finds that the decapsulated payload is neither IPv4 nor IPv6, records one analyzer violation in analyzer.log, non-IP packet in GTPv1, and does not pass the payload on, so no inner connection or second tunnel appears. The violation does not identify the payload as a tunnel.

Three independently written implementations, each forwarding only a payload it recognises as IP, therefore all miss the bare-nested form. This is also why the re-parse keys on the raw bytes rather than on the class the default parser assigned (Section 5.1). A realistic outer G-PDU makes Scapy guess the nested bytes as PPP, not Raw, and an earlier version of the rule that only re-read Raw payloads missed exactly this realistic case until a probe with a valid outer message type exposed it, a small instance of the paper's own thesis that testing against realistic wire bytes finds what in-memory tests do not. paper/REPRODUCE.md Part D reproduces the whole check in a few commands. Extending it to closed vendor stacks is the obvious next step and is not done here.

## 7. Implementation notes

- The corpus generator (attacker/generate_attacks.py) builds all six traffic classes (five malicious plus benign) as Scapy packet-construction functions, seeded with random.Random(seed) for full reproducibility. It supports both --write (a packet capture with aligned JSON labels, for offline scoring) and --send (live raw-socket transmission).

- The detector (detector/gtpu_detector.py) shares one rule engine and one metrics collector (detector/metrics.py) between its live mode (sniffing an interface) and its pcap mode (offline replay). It prints either human-readable or JSON output per finding, plus a summary block on exit, with classification metrics when ground-truth labels are supplied.

- The realistic benign generator (attacker/benign_traffic.py) produces the twelve-category benign corpus of Section 6.1, including the awkward Unstructured-PDU and handover categories.

- The naive baseline (detector/baselines.py) re-implements the four rules using only Scapy's default parsing, holding every design choice fixed except the inner re-parse, so the comparison in Section 6.1.3 is like-for-like.

- The evasion suite (attacker/evasions.py) lists 11 crafted evasions, each tagged with its expected outcome and checked in CI (Section 6.1.5).

- The evaluation harness (eval/benchmark.py, eval/run_eval.py, eval/report.py) chains corpus generation, robust and naive scoring, the false-positive analysis, the multi-seed run, and the evasion suite into one reproducible command, then writes the markdown report. Every number in Section 6.1 comes from a single python3 eval/run_eval.py.

- make live-score writes the exact corpus that make attack sends, with per-packet labels, and scores it through the detector's pcap mode with the same --core-ips and --gnb-ips as the live detector. The per-packet scores in Section 6.2 come from it.

## 8. Case study: the gap between "passes every test" and "works on the wire"

Before this investigation the offline test suite was fully green: 7 of 7 unit tests passing, and perfect precision, recall, F1, and false-positive rate on the labelled corpus. Bringing the live Docker lab up for the first time, which the project's own README treats as an equally important way to run it, exposed five real bugs (items 1 to 5 below). Two more (items 6 and 7) surfaced later, during revision, when a labelled live run was scored packet by packet. At that point the offline suite again passed in full, with 21 of 21 unit tests and the same perfect scores. Each defect is documented with full root-cause analysis and diffs in FINDINGS.md.

- The core image never installed mongosh, which both the entrypoint's health check and the upstream subscriber-provisioning tool depend on. Bring-up hung forever with no error at all, and every container still showed healthy in docker compose ps.

- The entrypoint's config-rebind logic used a regex that did not account for YAML's - address: list syntax, so it never matched. Because Python's re.sub() does not raise an error on zero matches, the script printed a confident and specific but wrong success message ("[core] bound NGAP+GTP-U to 10.10.10.10 ...") while the AMF and UPF stayed bound to internal loopback addresses that the RAN could not reach.

- A helper script (nr-binder) lost its executable bit while crossing a multi-stage Docker COPY --from=build.

- The most consequential one: the traffic generator's --send path called Scapy's sendp(), a Layer-2 send, on packets that were never given an Ether() layer. This does not raise an error. It quietly writes the raw IP bytes onto the wire, where any listener reads the first 14 bytes as a bogus Ethernet header. Every live-fired attack packet, for the whole history of the project up to this work, was malformed at the link layer. The fix, using Scapy's send() (Layer 3, so a real Ethernet header is added before the frame goes out), is two lines, but until then the project's own "fire abuse and watch detections" demo had never actually worked.

- The network_mode: "service:core" setting in docker-compose.yml, the very thing that makes the detector's tap point realistic (Section 4.3), binds to a specific container instance's namespace at attach time, not to a service name. Recreating core, which is needed to apply fixes 1 and 2, silently detached the detector and required an explicit --force-recreate to reattach. This is an easy trap to hit repeatedly while iterating. scripts/lab_up.sh does the reattach on a full bring-up, but make lab-up does not call that script, so after a manual core rebuild it is still a manual step.

- The UE data path never worked, and the documented check for it passed anyway. Nothing in the core container configured the UPF's TUN device ogstun: it had no address and its link was down. The UPF decapsulated each uplink GTP-U packet and then logged ogs_tun_write() failed, so no UE packet ever reached the data network. The documented check, docker compose exec ran ./build/nr-binder 10.45.0.2 ping -c3 8.8.8.8, still got replies. nr-binder works by preloading ./libdevbnd.so, a path relative to the current directory. Run from /ueransim as documented, the loader could not find it, printed an error above the ping output, and ran ping unbound. The ping left ran's eth0 from 10.10.10.20 by Docker's default route and never touched the tunnel; a tcpdump on the core saw zero GTP-U while it ran. core/entrypoint.sh now gives ogstun the address 10.45.0.1/16, brings it up, and NATs the UE pool out of eth0, and the check is now docker compose exec ran ping -I uesimtun0 -c3 8.8.8.8 (Section 6.2). The live detection results did not depend on this path, because the attacker is not a UE (Section 4.3).

- Rule R2 handed a TEID to whichever source sent last, and keyed it on the TEID alone. On any change of source the old code moved ownership to the new source before deciding whether to alert. In the live corpus every packet uses TEID 0x1, so spoofs interleave with the gNB's own packets on one tunnel. The first spoof was flagged but made the rogue the owner. The gNB's next benign packet was then flagged (16 false positives), and a spoof straight after another spoof matched the new owner and raised nothing (15 false negatives). The TEID-only key was a second fault: in this lab the real session's downlink TEID was also 0x1, the number the corpus sends to the UPF, and a TEID-only key puts both tunnels under one owner. R2 now keys ownership on (receiving address, TEID) and lets the first source win (Section 5.2). Three new unit tests and an extended baseline test fail on the old code and pass on the new, and make test now gives 24 passed, up from 21. The offline results in Section 6.1 are unchanged by the fix.

None of the first six were reachable from make test or make eval. Findings 1, 2, 3, 5 and 6 live entirely in Docker and container bring-up code, or in a manual check, that the offline path never runs. Finding 4 is the sharpest example of why this matters even for code the offline path does exercise indirectly. The packet-construction logic in generate_attacks.py is shared between the --write path, used by every offline test and the headline numbers in Section 6.1, and the --send path, used only live. The offline path never calls sendp() or send(), so a bug that lived only in how packets are transmitted, not in how they are built, was completely invisible to a result of 100% precision, 100% recall, and 0% false-positive rate.

Finding 7 is different. It is the only defect in detection logic, and both make test and make eval run that code. The offline inputs never produced the sequence that triggers it. The benchmark corpus gives every spoof its own victim TEID, emits all the victims first, and has a single receiver, so on each TEID the legitimate source is always seen first and the spoof second, with nothing after it. On that input "first source wins" and "last source wins" give the same verdicts, and the TEID-only key never meets two receivers. The original R2 unit test sent one legitimate packet and one spoof and checked only that the spoof was flagged. The live corpus interleaves spoofs and legitimate packets on one TEID, and both faults showed at once. Running the code was not enough here; the tests also needed an input in which spoofs and legitimate packets share one tunnel.

The general lesson is this. For any tool whose whole value is that it correctly reads or produces real network traffic, whether a passive detector, a protocol fuzzer, a traffic generator, or an IDS/IPS rule engine, a test suite based on packet captures or in-memory objects is necessary but not sufficient. The bug class that matters most for these tools, such as missing link-layer framing, a config writer that quietly does nothing, or a parser that discards the very bytes you are trying to detect, is by construction invisible to any test that never actually puts bytes on, or reads them off, a real wire. The project's own R1 finding (Section 5.1) and the seven findings here are the same failure mode found at two different layers of one codebase: code that looks correct, and in R1's case is even provably correct against an in-memory unit test, but quietly does the wrong thing, or nothing, once real bytes and real traffic are involved. We take this as an argument that, for security tooling which touches real wire traffic, running the live path end to end is not an optional demo but a necessary validation step that a passing offline suite cannot replace.

## 9. Limitations and threats to validity

- The corpus is synthetic, and the class balance is not realistic. The benign corpus now spans twelve categories, two of them chosen to be awkward for the detector, and it is no longer a single flow shape. It is still synthetic, though, and its roughly 55% benign share does not reflect how overwhelmingly benign real N3 traffic is. A measured false-positive rate of 0.0 on this corpus is far more informative than the earlier single-flow measurement, but it does not put a bound on the false-positive rate under production class imbalance, or against unusual-but-legitimate handset behaviour outside the twelve categories. Testing against captured operator traffic remains future work, and is limited by the legal and privacy issues in Section 11.

- R1's check is still a heuristic, now hardened but not cryptographic. _looks_like_gtp() together with the length-field and routable-inner-IP guards (Section 5.1) defeats the benign collisions and the evasions in Section 6.1.5. An attacker who fully understands the guard could still, in principle, craft a nested tunnel that passes all three checks yet is shaped to confuse some downstream parser differently from ours. That residual surface is untested and is inherent to any byte-pattern heuristic.

- R2's blind spots are documented, not closed. The known-gNB allowlist (Section 5.2) removes the handover false positives measured in Section 6.1.4, but a rogue that forges a legitimate gNB's source IP, or that claims a TEID never seen before, still evades R2 (Section 6.1.5). Both need control-plane correlation, which is out of scope for a user-plane-only tap. First-source-wins ownership also has no TEID ageing or expiry, so TEID recycling after a session ends, in a long-running deployment, is an unmeasured source of false positives. Without the gNB allowlist, ownership never moves, so a handed-over session raises R2 on every packet it sends from the new gNB.

- R3 works on ports and protocols, and is evaded by moving a control protocol off its usual port (Section 6.1.5). Closing this would need deep payload classification.

- R4 depends entirely on correct manual configuration of --core-ips. A core function outside the configured set is invisible (Section 6.1.5), and a wrong entry quietly disables the rule instead of raising an error.

- R1's re-parse recovers one nested header that forwards a routable inner packet, which is the construction the corpus contains. It does not catch two others. An attacker who nests twice defeats it. And the zero-length nested G-PDU behind CVE-2021-45462 carries no inner packet, so the length gate rejects it by design and, in the bare form, no rule fires; in the IP-wrapped form R3 still flags it as GTP-U inside the tunnel.

- IPv6 coverage is partial. R1's nesting check now handles an IPv6 inner tunnel (Section 6.1.5) and R3's port checks apply to IPv6 inner packets, but R3's SCTP check and R4 still inspect only IPv4 inner packets, so SCTP smuggling or core targeting inside an IPv6 inner packet is not yet detected.

- The setup is one subscriber, one IPv4 PDU session, and the test PLMN only. Unstructured and Ethernet PDU session types were not exercised live; the Unstructured case appears only as synthetic payloads in the offline corpus. The handover scenario is modelled synthetically with two gNB IPs, not run against a live multi-gNB core, and no roaming or network-slicing scenarios were exercised.

- The live attacker is not a rogue gNB with its own session. It writes crafted GTP-U onto the core's eth0 from inside the core's own namespace (Section 4.3), standing in for an on-path injector. Whether the UPF acts on those frames was not measured; the live results describe what the detector sees on the interface, not how the UPF behaves.

- Performance was measured on a single core, on one lab VM, with no other load. The figure of about 1,450 packets per second (Section 6.1) reflects the real cost of dissecting and checking every packet, but it is not validated against production N3 packet rates, and no multi-core or sustained-load testing was done.

- The live-run results (Section 6.2) come from two live runs of one seeded corpus, one before and one after the R2 fix, not a repeated set of trials, and are not directly comparable in size to the offline corpus (Section 6.3 explains what is and is not comparable). That corpus sends every packet on one TEID to one receiver. In both runs the live findings stream matched an offline pass over the same packets finding for finding.

## 10. Future work

- Stateful TEID lifecycle tracking with explicit expiry, to address the R2 limitation above.

- IPv6 inner-traffic coverage for R3 and R4.

- Growing the evasion suite (Section 6.1.5) from 11 hand-written cases into an automated fuzzer that mutates traffic around each rule's decision boundary, and closing the blind spots that have a user-plane-only signal (for example, classifying R3 payloads independently of port).

- A multi-gNB or roaming topology, to exercise TEID reuse across handovers for real rather than synthetically.

- The machine-learning extension sketched in docs/ARCHITECTURE.md: treating each rule's Finding output from evaluate() as a feature vector for a downstream classifier, instead of a final yes/no severity signal.

- Automating the recreate-and-reattach step from Finding 5 (FINDINGS.md) beyond the full bring-up in scripts/lab_up.sh, either as a make redeploy target or as a Compose health-check dependency, so it stops being a manual step after a core rebuild.

## 11. Ethics and responsible use

Everything in this project targets a self-contained lab on a private 10.10.10.0/24 Docker network, under the test PLMN 999/70, which is reserved for testing and is not a real operator's identity. The generator in attacker/ exists only to exercise the detector inside this isolated lab. It must not be pointed at any network the operator does not own and is not explicitly authorised to test. Generating GTP-U abuse traffic against a live mobile operator's infrastructure is illegal in most jurisdictions, and it was never done, attempted, or intended in this work. Every result in this paper was produced inside the isolated Docker lab described in Section 4. The only traffic that left the lab was the benign ICMP of the UE data-path check (Section 6.2), NATed to 8.8.8.8; no generated abuse traffic left it.

## 12. Conclusion

gtpu-abuse-lab shows that meaningful, reproducible research on GTP-U tunnel abuse does not need a closed commercial appliance or access to real operator infrastructure. A small, fully open stack (a real 5G core and radio simulator, a rule-based passive detector, and a labelled corpus generator, all in Docker) is enough to produce rigorous, reproducible detection numbers (Section 6.1) and to validate them against genuinely live, transmitted traffic (Section 6.2). The detector's central technical contribution (Section 5.1) is a specific and generalisable robustness requirement for passive GTP-U inspection: default parsing libraries can quietly discard the exact structure you are trying to detect, in a way that in-memory unit tests do not reveal. The project's own deployment history (Section 8) turned out to be a second, independent example of the same lesson at a different layer, and a concrete argument that, for security tooling which touches real wire traffic, running the live path end to end is not an optional demo but a necessary validation step that a passing offline test suite cannot substitute for.